REAGEN™

No-Mel Milk Total Protein Assay Kit

RND99031

### Product Description

REAGEN™ No-Mel Milk Total Protein Assay Kit is a colorimetric assay for the determination of protein in milk, powdered milk, cream, ice cream and chocolate drink.

The unique features of the kit are:

- Rapid and simple method (modified version of AOAC method) less than 10 minutes.

- Minimal sample prep.

- Highly accurate and reproducible.

- Not influenced by the presence of melamine in samples (<2 mg/mL in milk).

### Procedure Overview

The No-Mel Milk Total Protein Assay Kitmeasures the changes in soluble dye concentration caused by the presence of protein in dairy samples. The color change caused by consumption of the dye (measured by the absorbance at 490 nm) is related to the amount of protein present in the sample.

### Kit Contents, Storage and Shelf Life

REAGEN™ No-Mel Milk Total Protein Assay Kit has the capacity for 96 determinations or testing of 42 samples in duplicate (using 12 wells for standards).The shelf life is 6 months when the kit is properly stored.

| Kit Contents | Amount | Storage |
| --- | --- | --- |
| Microtiter Plate | 96-well plates (8 wells x 12 strips) | Room Temperature |
| Calibration Standard: (protein equivalent concentration): Std 1:0% protein (white cap tube) Std 2:1% protein (yellow cap tube) Std 3:2.5% Protein(orange cap tube) Std 4:2.8% protein (pink cap tube) Std 5:3.1% protein(purple cap tube) Std 6:4% protein (blue cap tube) | 1.6ml 1.6ml 1.6ml 1.6ml 1.6ml 1.6ml | Room Temperature |
| Dye Reagent | 2 X 26ml | Room Temperature |

### Dairy Sample Types

- Homogenized Milk

- Low-fat Milk

- Milk Powder

- Cream

- Ice Cream

- Chocolate Drink

### Required Materials Not Provided With the Kit

- Spectrophotometer (digital instrument preferred) or microtiter plate reader (with 490 nm absorbance filter)

- 1 cm cuvettes (glass, quartz or polystyrene) or micro plate

- Micro centrifuge tubes (polypropylene)

- Micro centrifuge

### Warnings and Precautions

- Do not use the kit past the expiration date.

- Do not intermix reagents from different kits or different lots.

- Try to maintain a laboratory temperature of 20°–25°C (68°–77°F).

- Avoid exposure of dye reagents to light.

- Make sure you are using only distilled or deionized water since water quality is very important.

- When pipetting samples or reagents into an empty microtiter plate, place th

### Sample Preparation

Milk and Other Liquid Dairy Samples

- Liquid samples can be tested directly without sample processing.

- Be sure samples are properly stored. In general, samples should be refrigerated at 2-4°C for no more than 1-2 days.

Dry Milk

- Reconstitute dry milk with appropriate volume of water according to the product label instruction, e.g. 9 -10 mL of water per gram of milk powder.

- Mix thoroughly for at least 30 minutes to dissolve (preferably mix overnight).

Note :Dilution Factor = mL of water added per gram. For example, adding 10 mL of water to 1 gram of milk powder gives a dilution factor of 10.

### Testing Protocol

- Dairy samples: Dilute 70 µL of sample (such as milk) into exactly 1.25 mL dye reagent into a microcentrifuge tube. Also prepare duplicate samples of “blank” samples where 70 µL of water is added to the dye reagent instead of dairy sample.

- Vortex mix the dairy samples for 30 seconds.

- Centrifuge the samples at room temperature for 4 minutes at 11,000 rpm.

- Transfer 0.7 mL of supernatant to a clean microcentrifuge tube. Avoid both the pellet at the bottom of the tube and lipids at the top surface of the sample (remove the supernatant from the middle). The sample is ready for the assay.

- Transfer (in duplicate) the appropriate amounts of purified water and the sample to the cuvette or microplate (see table below).

- Measure the absorbance at 490 nm.

| Sample Holder | Volume Supernatant/Purified Water | Volume dH2O | Total Volume |
| --- | --- | --- | --- |
| microplate | 10 μL | 290 μL | 300μL |
| 1 mL cuvette | 33 μL | 967 μL | 1000 μL |
| 3 mL cuvette | 100 μL | 2900 μL | 3000 μL |

### Calculation of curve Concentration

- Calibration curve: Label 6 clean microfuge tubes 1-6.Transfer exactly 1.25 mL of each

Calibration Standard into each tube, starting with the white cap tube in #1, yellow cap tube in #2, etc.Add 70 µL of water to each tube (see table below).

| Tube # | Volume Standards | Volume dH2O | Total Volume |
| --- | --- | --- | --- |
| 1 | 1.25 mL white cap | 70 µL | 1.32 mL |
| 2 | 1.25 mL yellow cap | 70 µL | 1.32 mL |
| 3 | 1.25 mL orange cap | 70 µL | 1.32 mL |
| 4 | 1.25 mL pink cap | 70 µL | 1.32 mL |
| 5 | 1.25 mL purple cap | 70 µL | 1.32 mL |
| 6 | 1.25 mL blue cap | 70 µL | 1.32 mL |

- Mix the calibration standards by vortexing for 30 seconds. The standards are ready to use.

- Transfer (in duplicate) the appropriate amounts of each standard (beginning with tube 1 and ending with tube 7) to the cuvette or microplate (see table below).

| Sample Holder | Volume Supernatant/Purified Water | Volume dH2O | Total Volume |
| --- | --- | --- | --- |
| microplate | 10 μL | 290 μL | 300μL |
| 1 mL cuvette | 33 μL | 967 μL | 1000 μL |
| 3 mL cuvette | 100 μL | 2900 μL | 3000 μL |

- Measure the absorbance at 490 nm.

Note: Although a calibration curve is not formally required to determine protein concentration using the No-Mel Milk Total Protein Assay Kit (see “Data Analysis” section below), it is useful to confirm the accuracy of spectrophotometers and plate readers in the determination of milk protein concentration in individual labs.

For each calibration standard, plot the average absorbance at 490 nm as a function of protein

concentration (percent protein). This plot should provide a tight linear curve with an R2 value >0.99 for the linear fit.

### Calculation of Protein Concentration

Protein concentration can easily be determine with the following equation

% protein = 7.22 x (1 – AS/AB),

where “AS” is the average absorbance for the sample and “AB” is the average absorbance of the blank samples (in which water was added to the dye instead of dairy sample).

A special program with Excel functionality, Total Protein Analysis Program in Excel is available upon request to evaluate the test results. Please contact your local distributor for further information.

REAGEN LLC

Add.: 102CommerceDr.Unit8,Moorestown,NJ08057

Tel.:856-727-0250         FAX.:856-727-0251

E-mail:reagenkits@gmail.com  reagenllc@gmail.com

Web.: www.reagen.us