REAGEN™

Deoxynivalenol (DON) Column Kit

RNM98013

### Product Description

- REAGEN™ Deoxynivalenol (DON) Column Kit contains immunoaffinity columns for the convenient purification and enrichment of Deoxynivalenol (also known as Vomitoxin or DON) from a variety of food and feed samples for downstream HPLC and ELISA analysis. DON is a naturally-occurring fungal toxin that can be present in many different types of food. Contamination of food with DON can cause a variety of serious illnesses. The columns contain immobilized antibodies which bind DON. When food sample extracts are passed through the columns, DON will specifically bind to the column while the other substances pass through. The analyte is then eluted from the column using methanol.

### Procedure Overview

REAGEN™ immunoaffinity columns use a simple procedure to enrich DON from food samples. A liquid extract prepared from the sample is passed through the column to bind the DON. After washing the column with a wash solution, the DON is eluted using methanol.The enriched samples are suitable for subsequent analysis by HPLC or ELISA methods. Each column is disposable and should only be used once.

### Kit Contents, Storage and Shelf Life

REAGEN™ Deoxynivalenol (DON) Column Kit contains 25 columns. Store the kit at room temperature (18 - 25°C). The shelf life is 18 months when the kit is properly stored.

| Kit Contents | Amount | Storage |
| --- | --- | --- |
| columns | 25 | Room temp |

### Required Materials Not Provided With the Kit

- 50 mL plastic screwcap tubes

- Deionized water

- Acetonitrile

- Methanol

- Pipettes and pipette tips

- Column stand or vacuum manifold

- Homogenizer, food processor, or grain mill

- Fluted filter paper (Whatman 40)

- Glass microfiber filter

- Nitrogen evaporator or vacuum centrifuge

### Warnings and Precautions

- Do not use the kit past the expiration date.

- Try to maintain a laboratory temperature of 20°–25°C (68°–77°F).

- Use only distilled or deionized water to prepare wash solutions; water quality is very important.

- Do not allow the columns to be stored at temperatures above 30°C.

SAMPLE PREPARATION&CHROMATOGRAPHY PROCEDURES

Be sure samples are properly stored. In general, samples should be refrigerated at 2-4°C for no more than 2-3 days. Chilled or frozen samples should be equilibrated to room temperature 20°–25°C (68°–77°F)before use.

Corn

Extraction

- Transfer exactly 2.5 g of ground sample to a 50 mL plastic tube. Note: Kernels should finely ground such that the sample particles are less than 1 mm in size.

- Mix with 20 mL deionized water.

- Vortex for 30 minutes.

- Add 22.5mL deionized water

- Pour diluted extract into fluted filter paper and collect filtrate in a clean vessel.

- Filter the dilute extract through a glass microfiber filter into a clean tube.

Chromatography

- Remove the upper and lower caps from the DonPure column. Discard the liquid from the top portion of the column.

- Pass 1.5 mL filtered dilute extract completely through the column at a rate of about 1 to 2 drops per seconds until air comes through the column.

- Pass 1 mL deionized water through the column at a rate of about two drops per second.

Elute the Deoxynivalenol from the column using 1 mL HPLC grade methanol into a clean tube. Use a flow rate of one drop per second.

REAGEN LLC

Add.: 102CommerceDr.Unit8,Moorestown,NJ08057

Tel.:856-671-1789         FAX.:856-858-5434

E-mail:reagenkits@gmail.com  reagenllc@gmail.com

Web.:www.reagen.us

- Evaporate to dryness using a nitrogen stream or vacuum centrifuge.

- Redissolve in 0.1 mL solvent (87% water/ 13% acetonitrile)

Note: Dilution factor: 1.2

The sample is now ready for analysis by HPLC or ELISA.

### Wheat

Extraction

- Transfer exactly 2.5 g of ground sample to a 50 mL plastic tube. Note: Grain should finely ground such that the sample particles are less than 1 mm in size.

- Mix with 20 mL deionized water.

- Vortex for 30 minutes.

- Add 22.5mL deionized water

- Pour diluted extract into fluted filter paper and collect filtrate in a clean vessel.

- Filter the dilute extract through a glass microfiber filter into a clean tube.

Chromatography

- Remove the upper and lower caps from the DonPure column. Discard the liquid from the top portion of the column.

- Pass 1.5 mL filtered dilute extract completely through the column at a rate of about 1 to 2 drops per seconds until air comes through the column.

- Pass 1 mL deionized water through the column at a rate of about two drops per second. Allow to flow until air comes through the column.

- Elute the Deoxynivalenol from the column using 1 mL HPLC grade methanol into a clean tube.Use a flow rate of one drop per second.

- Evaporate to dryness using a nitrogen stream or vacuum centrifuge.

- Redissolve in 0.1 mL solvent (87% water/ 13%ACN).The sample is now ready for analysis by HPLC or ELISA.

Note: Dilution factor:1.2

RELATED PRODUCTS

| - Product | - Catalog Number |
| --- | --- |
| - REAGEN™DON ELISA Test Kit | RNM98004 |
| - REAGEN™Total Aflatoxin fluorescence Column Kit | - RNM98012 |
| - REAGEN™Total Aflatoxin Rapid Column Kit | - RNM98011 |
| - REAGEN™M1 Aflatoxin rapid column Kit | - RNM98010 |